## Maritime Incident:

## Grounding and Salvage of Dredger Bhagvati Prem.

#### Casualty Vessel and Location

The vessel involved was the Trailing Suction Hopper Dredger (TSHD) 'Bhagvati Prem'. This type of vessel is specialized for maintaining or deepening shipping channels by sucking sediment from the seabed. The dredger was operated by a private entity, engaged in dredging activities near the New Mangalore Port (NMPT).

The grounding incident occurred on October 27, 2019. The vessel was driven ashore onto Surathkal Beach (sometimes referred to as Surakal Beach), which is located north of Mangalore, Karnataka State, India (not Bangalore, which is inland).

#### Cause of Grounding

The grounding was a direct consequence of the severe weather conditions generated by Super Cyclonic Storm Kyarr, which was active in the Arabian Sea during that period.

- Storm Conditions: The cyclone, though its center was far from the Karnataka coast, generated extreme weather conditions, including exceptionally high waves, severe sea swells, and strong onshore winds.

- Anchor Failure and Drift: The dredger, which was likely anchored or operating near the New Mangalore Port, was overwhelmed by the tremendous force of the waves and wind. Its anchor system failed, causing the vessel to drift uncontrollably towards the shore.

- Grounding: The massive vessel was pushed onto the beach and ran aground on the sandy/rocky shore near Surathkal. The dredger was stranded very close to the coastline, making it accessible but also immediately vulnerable to further damage from the continuous wave pounding.

#### Immediate Concerns and Crew Safety

The initial aftermath presented significant challenges regarding safety and environment:

- Crew Status: The vessel had a complement of approximately 18 crew members onboard. Although the grounding was violent, all crew members were reported to be safe, as the vessel remained relatively stable on the sandy shore. The crew was evacuated from the stranded dredger shortly after the incident, primarily using local rescue means and Coast Guard assistance.

- Environmental Risk: The immediate environmental concern was the potential for a fuel oil spill. As a large, powered vessel, the 'Bhagvati Prem' carried several tons of Heavy Fuel Oil (HFO) and diesel for its operations. Any breach in its fuel tanks due to hull damage from the grounding posed a significant threat to the sensitive Surathkal coastline and local marine life.

#### The Salvage Challenge and Initial Strategy

The salvage operation was complex and prolonged, requiring specialized equipment and favorable weather conditions:

- Vessel Condition: After the grounding, an initial inspection revealed that the dredger had sustained hull damage, which was the main reason for the subsequent actions.

- Refloating Attempt: The first objective was to refloat the vessel, which was attempted by professional salvagers. This involved lightening the vessel by removing unnecessary weight (ballast water, non-essential equipment) and using high-powered towing tugs, typically synchronized with high tide. This initial attempt to refloat was successful.

#### The Second Grounding and Stabilization

The refloating success was short-lived, leading to a crucial, intentional maneuver:

- Post-Refloating Distress: Upon being pulled free from the beach, it was discovered that the dredger was taking on water at an alarming rate due to the unrepaired hull damage sustained during the grounding and the subsequent wave pounding. The vessel's stability was compromised, making it susceptible to sinking in deeper water.

- Intentional Beaching: To prevent the TSHD 'Bhagvati Prem' from sinking completely in the navigational channel or deeper sea, the decision was made to intentionally ground (beach) the vessel again in a controlled manner near the Surathkal shore. This maneuver was crucial as it stabilized the vessel, ensured the hull was resting on a solid bottom, and allowed salvagers access to conduct temporary repair work and remove remaining oil.

#### Final Resolution and Regulatory Scrutiny

The dredger remained intentionally aground for an extended period while a comprehensive salvage plan was executed:

- Oil Removal: The priority immediately shifted to removing all heavy fuel oil and diesel from the vessel's tanks to eliminate the environmental pollution risk, before any major permanent repair or second refloating attempt was made. This crucial step was monitored by the Indian Coast Guard.

- Final Salvage: The long-term salvage operation involved specialized repairs to the damaged hull plating while the vessel was beached, followed by a final, successful refloating and towage to a safe harbour for permanent repairs.

- Inquiry: The incident led to a regulatory inquiry by the New Mangalore Port authorities and the Directorate General of Shipping. This inquiry would focus on why the dredger did not take adequate shelter in the port well ahead of the predicted cyclonic storm, and whether the emergency procedures were followed correctly.

The grounding of the dredger 'Bhagvati Prem' was a complex maritime incident where the damage from the cyclone-induced grounding necessitated a risky but successful intentional second beaching to prevent a disastrous sinking and large-scale fuel spill in the sensitive coastal waters of Karnataka.